Fort Lupton Intersection README

Overview

The Fort Lupton Intersection deployment is led by the Air Toxics and Ozone Precursors Program (ATOPs) within the Air Pollution Control Division (APCD) of the Colorado Department of Public Health and Environment (CDPHE). The deployment was initiated to respond to emissions of benzene from a broken orange pipeline along County Road 20 in Fort Lupton. This README file explains the instruments used, compounds detected, units of measurement, and specifications relevant for data interpretation.

Measurement Specifications

BTEX (Benzene, Toluene, Ethyl Benzene, Xylenes)

| Instrument Manufacturer | Pollution Analytical Equipment |
| --- | --- |
| Instrument Model | PyxisGC BTEX |
| Compounds Detected | Benzene, Toluene, Ethyl Benzene, Xylenes (BTEX) |
| Units | ppbV, ppbV, ppbV, ppbV |
| Detection Limits (in units) | 0.05, 0.05, 0.25, 1 |
| Sampling Resolution | 597 seconds (9 minutes, 57 seconds) |
| Notes | Sampling onto the preconcentrator occurs during the last 6 minutes of each sampling period. |

Health Guideline Value (HGV) / Level 1 Acute Exposure Guideline Levels (AEGL) Reference Guide

These concentrations are established by the EPA (AEGL) and CDPHE (HGV) for the compounds measured at this deployment that can cause acute health effects.

| Exposure Time | Benzene (ppbV) |
| --- | --- |
| 10 min | 130000 |
| 30 min | 73000 |
| 60 min | 52000*/9** |
| 4 hr | 18000 |
| 8 hr | 9000 |

*1 hour AEGL
**1 hour HGV