Fort Lupton Intersection README

Overview

The Fort Lupton Intersection deployment is led by the Air Toxics and Ozone Precursors Program (ATOPs) within the Air Pollution Control Division (APCD) of the Colorado Department of Public Health and Environment (CDPHE). The deployment was initiated to respond to emissions of benzene and methane from a broken orange pipeline along County Road 20 in Fort Lupton. This README file explains the instruments used, compounds detected, units of measurement, and specifications relevant for data interpretation.

Measurement Specifications

Methane (CH4)

| Instrument Manufacturer | Aeris Technologies, Inc. |
| --- | --- |
| Instrument Model | MIRA LDS |
| Compounds Detected | CH4, Ethane (C2H6), H2O |
| Units | ppmV, ppbV, ppmV |
| Detection Limits (in units) | 0.002, 0.5, N/A |
| Sampling Resolution | 1 second |
| Notes | CH4 reported as dry mixing ratio, water is measured and subtracted from signal. |

Benzene

| Instrument Manufacturer | Entanglement Technologies |
| --- | --- |
| Instrument Model | AROMA-VOC |
| Compounds Detected | Benzene |
| Units | ppbV |
| Detection Limits (in units) | 0.046 |
| Sampling Resolution | 10 minutes |
| Notes | Sampling period is 10 minutes, measurement pauses between samples to process data and can vary slightly. |

Health Guideline Value (HGV) / Level 1 Acute Exposure Guideline Levels (AEGL)

These concentrations are established by the EPA (AEGL) and CDPHE (HGV) for the compounds measured at this deployment that can cause acute health effects. Methane is explosive before it becomes toxic, and therefore is not included in this table.

| Exposure Time | Benzene (ppbV) |
| --- | --- |
| 10 min | 130000 |
| 30 min | 73000 |
| 60 min | 52000*/9** |
| 4 hr | 18000 |
| 8 hr | 9000 |

*1 hour AEGL
**1 hour HGV